Mobile Radio Systems 404

Mini Project

Jason Colman

09823264

Ethan Wren

Yep, some poof number

# Contents Page

Contents Page	2

Introduction	3

Task 1: Cumulative Distribution Function of a Rayleigh fading channel	4

Task 2: Find Maximum Level Crossing Rate	5

Task 3: Average Fade Duration	7

Task 4:	8

Appendix	9

Task 1 – Maple	9

Task 1 – Matlab	9

Task 2 – Maple	10

Task 2 – Matlab	11

Task 3 – Maple	12

Task 3 – Matlab	13

# Table of Figures

Figure 1: Cumulative Distribution Function for received signal	5

Figure 2: Level Crossing Rate for a Rayleigh Fading System. fd = 45Hz	6

Figure 3: Product of AFD and Maximum Doppler Shift	7

# Introduction

This mini project aims at studying a model of a multipath channel. When an unmodulated carrier wave, , is transmitted through a large collection of scattered objects such as an urban environment, the received signal at the antenna can be expressed as a cumulative sum of different phase shifted and attenuated scattered versions of the original signal. This can be expressed in the equation below:

(1)

Where  is the collection of N incident scattered waves on the antenna.  and  are random variables and  is the amplitude of the ith scattered wave. If we express  in its equivalent quadrature baseband form, such that

(2)

where  and

From here the amplitude fading  and the phase distortion  can be given by

(3)

(4)

These parameters  and  describe the fading characteristics of the channel. Both Maple and Matlab were used to complete the required tasks in this lab, as Matlab has superior plotting capabilities while Maple has strengths in algebraic manipulation and solving capabilities.

## Task 1: Cumulative Distribution Function of a Rayleigh fading channel

The fading characteristics of the channel given by  and  in equations 3 and 4 can be characterised by their associated probability density function (pdf).

(5)

(6)

gives the probability of the fade having a given amplitude, while  indicates that there is equal an equal probability of  for all phase shifts. Following this, the Cumulative Distribution Function (CDF) of the envelope is given by:

(7)

whereis the normalised threshold with respect to the rms value of , ie

and

Now, if we plot equation 7 with respect to  on the range of 30 to 10 dB we obtain the figure in Figure 1. Taking the difference of equation 7 evaluated at 10dB and at 30dB we see that the fading envelope fluctuates over this range with a probability of 99.8955%.

Figure 1: Cumulative Distribution Function for received signal

## Task 2: Find Maximum Level Crossing Rate

The crossing rate at a level R of a flat fading channel, NR, is defined as the expected number of times that the fading signal envelope  crosses the specific level, R, in a positive going direction.

(8)

where

is the joint pdf of the amplitude fading  and its derivative  and is given by:

for 	(9)

where

Substituting equations 8 and 9 then solving yields the crossing rate NR.

NR=2πfdρe−ρ2	(10)

where  is the maximum Doppler shift due to motion between the transmitter and the receiver and is given by

v is in ms-1

Now if we set the derivative of the crossing rate to zero and solve, we can determine the normalised threshold of the maximum crossing rate for a given instance. So taking the derivative of equation 10 yields:

(11)

then solving for ;

For the case where , and  we can plot the following LCR for a normalised threshold.

Figure 2: Level Crossing Rate for a Rayleigh Fading System. fd = 45Hz

Substituting , ,  (in ms-1) and  into equation 10 indicates a positive LCR of approximately 48 crossings per second.

## Task 3: Average Fade Duration

The average fade duration (AFD)  is defined as the average period of time for which the amplitude fading level is below the specified level R during each fade period. From definitions of LCR and AFD we can show that the probability of  can be given by.

(12)

Next we plot the product of the AFD and Doppler shift vs. the normalised threshold.

Figure 3: Product of AFD and Maximum Doppler Shift, for = 3dB, the AFD = 8ms.

Figure 3 indicates that there is a AFD of 8ms for a value of = 3dB.

## Task 4:

Yeeeeeaahhh!!! Put your shit here dude!!!

# Appendix

### Task 1 – Maple

> restart;

with(plots):

P[cdf]:=rho_dB->1-exp(-(10^(rho_dB/20))^2);

Warning, the name changecoords has been redefined

> plot(P[cdf](rho_dB),rho_dB=-30..10, color=black,labels=["rho(dB)","P(x<=R)"]);

> prob:=evalf(P[cdf](10)-P[cdf](-30));

### Task 1 – Matlab

%%%%%%%%%%%%%%%%%%%%%%%%%%

%%       Advanced Topics In Electronics		%

%%       and Communtications 404               	%

%%       Mini Project - Task 1                 		%

%%                                             		%

%%      Jason Coleman & Ethan Wren             	%

%%      09823264        Some faggy number      	%

%%      Created:        14 August 2003         	%

%%      Last Modified:  8 September 2003       	%

%%                                             		%

%%      Demonstrates the CBF of the Rayleigh   	%

%%      Fading Channel and shows that          	%

%%      the probability the rho being          	%

%%      between -30 and 30 dB is 99.98%        	%

%%                                             		%

%%%%%%%%%%%%%%%%%%%%%%%%%%

%-- Clear the workspace & close all plots

clear all;

close all;

%-- create rho vector

rho_dB = [-30:0.01:10];

%-- Calculate CDF Over Given Range

Pr = 1-exp(-(10.^(rho_dB/20)).^2);

%-- Plot Results & Demonstrate Probability as per Above

figure

plot(rho_dB,Pr,':');

title('Cumulative Distribution Function')

ylabel('P_r[x \leq R]')

xlabel('\rho (dB)')

gtext(['Fading envelope fluctuates with a probability of ' num2str(100*(Pr(end)-Pr(1)))])

### Task 2 – Maple

> restart;

with(plots):

f:=900E6:

v:=54/3.6:

c:=3E8:

N[R]:=rho->sqrt(2*Pi)*rho*fd*exp(-rho^2);

Warning, the name changecoords has been redefined

Differentiate to find Maximum

> Nprime[R]:=diff(N[R](rho),rho);

> Nprime[R]:=unapply(Nprime[R],rho);

Lets see if it works :)

> fd:=(v*f)/c;

> plot(N[R](rho),rho=0.01..3,numpoints=1000, color=black,axes=framed,labelfont=[SYMBOL],labels=[r,N]);

> Nprime[R](5);

> rho[max]:=simplify(evalf(solve(Nprime[R](rho)=0)));

> rho[max(dB)]:=20*log10(rho[max]);

Show Value

> evalf(N[R](rho[max]));

### Task 2 – Matlab

%%%%%%%%%%%%%%%%%%%%%%%%%%

%%       Advanced Topics In Electronics        	%

%%       and Communtications 404               	%

%%       Mini Project - Task 2                 		%

%%                                             		%

%%      Jason Coleman & Ethan Wren             	%

%%      09823264        Some faggy number      	%

%%      Created:        14 August 2003         	%

%%      Last Modified:  8 September 2003       	%

%%                                             		%

%%      Demonstrates the LCR of the Rayleigh   	%

%%      Fading Channel and shows that          	%

%%      for a carrier frequency of 900MHz      	%

%%      and a mobile station traveling at      	%

%%      54km/hr the maximum LCR will be 48     	%

%%      crossings per second at rho = -3dB     	%

%%                                             		%

%%%%%%%%%%%%%%%%%%%%%%%%%%

%-- Clear the workspace & close all plots

clear all;

close all;

%-- Define Some Constants

v = 54/3.6;

fc = 900E6;

c = 3E8;

f_d = v*fc/c;

%-- Create a rho vector

rho = [0.01:0.0001:3];

%-- Calculate the LCR for specified fd

Nr = sqrt(2*pi)*f_d*rho.*exp(-rho.^2);

%-- Make some plots

figure

plot(rho,Nr);

title(['LCR of a Rayleigh Fading Channel with f_d = ',num2str(f_d)])

xlabel('\rho')

ylabel('N_R')

ax=axis;

%-- Find the maximum of the curve

[y,x] = max(Nr);

%-- Demonstrate Maximum on Plot and add some text

hold on

plot(ax(1:2),[y y],'k:',[rho(x) rho(x)],ax(3:4),'k:')

text(rho(x)+.05,25,['\rho = ', num2str(20*log10(rho(x))),'dB'])

text(1,46,['N_R _m_a_x = ', num2str(max(Nr))])

### Task 3 – Maple

> restart;

with(plots):

N[R]:=rho_dB->(exp((10^(rho_dB/20))^2)-1)/(sqrt(2*Pi)*(10^(rho_dB/20)));

Warning, the name changecoords has been redefined

> logplot(N[R](rho_dB),rho_dB=-30..10,numpoints=1000, color=black,labelfont=[SYMBOL],labels=[r,"G(r)"],ytickmarks=4);

> rho:=-3;

show value

> f:=900e6:

c:=3e8:

v:=15:

f[d]:=(v*f)/c;

> evalf(N[R](rho)/f[d]);

### Task 3 – Matlab

%%%%%%%%%%%%%%%%%%%%%%%%%%

%%       Advanced Topics In Electronics        	%

%%       and Communtications 404               	%

%%       Mini Project - Task 3                 		%

%%                                             		%

%%      Jason Coleman & Ethan Wren             	%

%%      09823264        Some faggy number      	%

%%      Created:        14 August 2003        	%

%%      Last Modified:  8 September 2003       	%

%%                                             		%

%%      Demonstrates the product of AFD and    	%

%%      the Doppler shift for a Rayleigh       	%

%%      Fading Channel and shows that          	%

%%      an AFD of about 8ms occures at rho     	%

%%      being -3dB                             		%

%%                                             		%

%%%%%%%%%%%%%%%%%%%%%%%%%%

%-- Clear the workspace & close all plots

clear all;

%-- Define Some Constants

v = 54/3.6;

fc = 900E6;

c = 3E8;

f_d = v*fc/c;

%-- Calculate the AFD value at the required rho value

dB_val = -3;

Gamma_val = (exp((10^(dB_val/20))^2)-1)./(sqrt(2*pi)*(10^(dB_val/20)));

%-- create rho vector over a range

rho_dB = [-15:0.01:5];

%-- Calculate Gamma values over rho range

Gamma = (exp((10.^(rho_dB/20)).^2)-1)./(sqrt(2*pi)*(10.^(rho_dB/20)));

%-- Plot the Resutls

figure

semilogy(rho_dB,Gamma);

title('Product of AFD and Maximum Doppler Shift for a Rayleigh Fading Channel')

xlabel('\rho (dB)')

ylabel('\Gamma_R.f_d')

ax = axis;

%-- Show the AFD for a given rho value on the Plot.

hold on

plot(ax(1:2),[Gamma_val Gamma_val],'k:',[dB_val dB_val],ax(3:4),'k:')

text(dB_val,Gamma_val,['AFD \approx ' int2str(round(Gamma_val/f_d*1000)) 'ms '],...

'VerticalAlignment','bottom','HorizontalAlignment','right')

text(dB_val,Gamma_val,[' \rho = ', num2str(dB_val), 'dB'],...

'VerticalAlignment','top','HorizontalAlignment','left')